GUTS Data Acquired at GGAO

The following pages present data analysis results based on data acquired by the GUTS system while at GGAO.  The analysis is somewhat abbreviated due to the accelerated delivery schedule.  No formal ground tests were taken, and only 3 Lageos 1 and Lageos 2 passes were collocated.  While the GUTS system acquired its first pass, a small Starlette pass, on September 17, 2003, system development continued to impact system performance for the next several months.  System performance became relatively stable on the later part of December 2, 2003.

Three sections follow, starting with ground data, then satellite data, and finally collocated data.

# Ground Data

The GUTS system acquired 67 ground target data sets since December 2, 2003, primarily to calibrate satellite passes.  No formal ground tests were performed due to the accelerated delivery schedule.  Eight of the data sets were taken in geosynchronous mode.  The delays are two-way delays, so the apparent range error between targets 5009991 and 509992 would be (56.4158-56.4081)/2, or 3.85mm.

Ground Targets Used Since December 2, 2003

| Target Name | Target ID | No. of Data Sets | Mean Delay (meters) |
| --- | --- | --- | --- |
| Target-B | 5009991 | 7 | 56.4158 |
| Target-C | 5009992 | 43 | 56.4081 (5 sets edited) |
| Target-B Geosynchronous | 5009981 | 6 | 35.6250 |
| Target-C Geosynchronous | 5009982 | 1 | 35.6303 |
| Internal Cal | 5009901 | 9 | 56.1902 |
| Internal Cal Geosynchronous | 5009902 | 1 | 35.6531 |

Ground Target Data Sets Acquired Since December 2, 2003

| Date Time (GMT) | Observations | RMS (meters) | Delay (meters) |
| --- | --- | --- | --- |
| 5009991 – Ground Target B |  |  |  |
| 2003-12-02 03:33:11 | 1084 | 0.001601 | 56.415386 |
| 2003-12-02 05:05:51 | 1075 | 0.001494 | 56.413501 |
| 2003-12-02 06:05:30 | 1731 | 0.001293 | 56.411957 |
| 2003-12-02 07:09:48 | 962 | 0.001465 | 56.417484 |
| 2003-12-30 23:30:38 | 1034 | 0.001655 | 56.415602 |
| 2003-12-31 00:59:18 | 1057 | 0.001651 | 56.418068 |
| 2003-12-31 01:25:51 | 1266 | 0.001596 | 56.41829 |
| 5009992 – Ground Target C |  |  |  |
| 2003-12-02 03:30:11 | 1120 | 0.001421 | 56.416708 |
| 2003-12-02 05:08:48 | 1022 | 0.001429 | 56.413718 |
| 2003-12-02 05:44:14 | 61 | 0.001538 | 56.415633 |
| 2003-12-02 05:49:12 | 766 | 0.001449 | 56.412864 |
| 2003-12-02 05:54:31 | 872 | 0.001336 | 56.412055 |
| 2003-12-02 21:39:50 | 1357 | 0.001704 | 56.429154 |
| 2003-12-02 23:46:07 | 996 | 0.001561 | 56.412109 |
| 2003-12-03 00:41:12 | 927 | 0.001286 | 56.415772 |
| 2003-12-03 01:11:20 | 1058 | 0.001596 | 56.415247 |
| 2003-12-03 02:14:11 | 895 | 0.001387 | 56.413785 |
| 2003-12-03 05:12:05 | 1216 | 0.001547 | 56.415762 |
| 2003-12-07 22:41:14 | 1012 | 0.001633 | 56.416104 |
| 2003-12-07 23:30:04 | 1055 | 0.00159 | 56.414744 |
| 2003-12-08 00:39:54 | 771 | 0.001573 | 56.417432 |
| 2003-12-08 01:21:15 | 112 | 0.003667 | 57.088516 |
| 2003-12-08 01:22:16 | 938 | 0.003661 | 57.090545 |
| 2003-12-08 16:03:49 | 5859 | 0.003553 | 57.07439 |
| 2003-12-08 16:31:46 | 1021 | 0.003625 | 57.073768 |
| 2003-12-11 22:49:02 | 6411 | 314122.967 | 4289629.034 |
| 2003-12-11 23:01:49 | 1107 | 0.002206 | 56.419803 |
| 2003-12-12 00:40:40 | 1533 | 0.001556 | 56.414234 |
| 2003-12-12 02:13:32 | 979 | 0.001438 | 56.407468 |
| 2003-12-12 03:13:05 | 975 | 0.00149 | 56.407843 |
| 2003-12-12 04:13:46 | 1288 | 0.001563 | 56.412802 |
| 2003-12-12 05:37:44 | 1120 | 0.001585 | 56.412137 |
| 2003-12-15 22:25:04 | 2096 | 0.001556 | 56.410396 |
| 2003-12-16 00:10:15 | 1543 | 0.001675 | 56.410537 |
| 2003-12-16 00:47:48 | 2724 | 0.001694 | 56.409445 |
| 2003-12-16 01:41:20 | 1112 | 0.001739 | 56.408664 |
| 2003-12-16 03:39:04 | 1014 | 0.00174 | 56.419071 |
| 2003-12-31 22:13:22 | 1599 | 0.001496 | 56.43146 |
| 2003-12-31 22:34:11 | 1088 | 0.001419 | 56.431943 |
| 2004-01-07 22:50:33 | 1062 | 0.002237 | 56.387347 |
| 2004-01-07 23:24:38 | 1036 | 0.001417 | 56.388516 |
| 2004-01-08 00:06:03 | 1061 | 0.001394 | 56.388086 |
| 2004-01-08 01:13:40 | 1034 | 0.001486 | 56.390165 |
| 2004-01-08 02:23:30 | 1092 | 0.001572 | 56.391487 |
| 2004-01-09 23:32:18 | 164 | 0.0013 | 56.388722 |
| 2004-01-09 23:33:24 | 809 | 0.001316 | 56.388352 |
| 2004-01-10 00:56:42 | 1196 | 0.00154 | 56.398319 |
| 2004-01-10 01:28:57 | 1026 | 0.001384 | 56.395097 |
| 2004-01-10 23:15:19 | 1166 | 0.00139 | 56.386039 |
| 2004-01-11 00:39:03 | 946 | 0.00142 | 56.387387 |
| 5009982 – Ground Target C Geosynchronous |  |  |  |
| 2003-12-03 01:58:43 | 245 | 0.014176 | 35.622757 |
| 2003-12-31 21:28:51 | 320 | 0.01416 | 35.64273 |
| 2003-12-31 22:09:43 | 202 | 0.014619 | 35.642154 |
| 2004-01-10 00:14:57 | 462 | 0.012241 | 35.624634 |
| 2004-01-10 00:29:46 | 482 | 0.012691 | 35.622044 |
| 2004-01-10 00:52:58 | 105 | 0.015537 | 35.627681 |
| 5009981 – Ground Target B Geosynchronous |  |  |  |
| 2003-12-31 01:31:27 | 136 | 0.012775 | 35.624965 |
| 5009901 – Internal Calibration Target |  |  |  |
| 2003-12-02 06:01:31 | 1572 | 0.002328 | 57.234901 |
| 2003-12-02 07:18:24 | 945 | 0.003862 | 56.886064 |
| 2003-12-02 07:24:26 | 367 | 0.003795 | 56.42211 |
| 2003-12-02 07:30:10 | 2598 | 0.230652 | 56.203949 |
| 2003-12-02 07:35:51 | 1978 | 0.008363 | 56.443646 |
| 2003-12-02 07:40:05 | 553 | 0.009768 | 56.41532 |
| 2003-12-03 01:17:08 | 90 | 0.00901 | 55.235178 |
| 2003-12-03 01:17:30 | 776 | 0.382815 | 55.633268 |
| 2003-12-03 01:19:11 | 801 | 0.00652 | 55.237451 |
| 5009902 – Internal Calibration Target Geosynchronous |  |  |  |
| 2003-12-03 02:10:51 | 664 | 0.01282 | 35.653103 |

# Satellite Data

The GUTS system acquired a total of 88 satellite passes while tracking from GGAO.  19 different satellites were tracked from September 17, 2003 through January 11, 2004.  Data quality markedly improved on the night of December 2, 2003, after some troubleshooting and corrections.  The system acquired 57 passes after that time.

Satellites Tracked at GGAO

| Name | SIC | COSPAR | Number of Passes | After 12/02/2003 |  |
| --- | --- | --- | --- | --- | --- |
| Ajisai | 1500 | 8606101 | 10 | 4 |  |
| BEC | 317 | 6503201 | 6 | 6 |  |
| Champ | 8002 | 0003902 | 2 | 2 |  |
| Envisat-1 | 6179 | 0200901 | 5 | 4 |  |
| ERS-2 | 6178 | 9502101 | 4 | 3 |  |
| Etalon-1 | 525 | 8900103 | 3 | 3 |  |
| Etalon-2 | 4146 | 8903903 | 1 | 0 |  |
| GFO-1 | 8501 | 9800701 | 3 | 1 |  |
| Glonass-84 | 9084 | 0006302 | 2 | 0 |  |
| Glonass-87 | 9087 | 0105302 | 2 | 0 |  |
| Glonass-89 | 9089 | 0206001 | 4 | 4 |  |
| GPS-35 | 3535 | 9305401 | 2 | 2 |  |
| Jason | 4378 | 0105501 | 7 | 6 |  |
| Lageos-1 | 1155 | 7603901 | 5 | 4 |  |
| Lageos-2 | 5986 | 9207002 | 8 | 6 |  |
| Larets | 5557 | 0304206 | 3 | 2 |  |
| Starlette | 1134 | 7501001 | 7 | 0 |  |
| Stella | 643 | 9306102 | 4 | 4 |  |
| Topex | 4377 | 9205201 | 10 | 6 |  |

The following table displays some SPAS processing results retrieved from the MySQL database.  A ‘G’ following the Date of Data indicates that the pass was acquired in geosynchronous mode.

Passes Acquired After December 2, 2003

| Date of Data (GMT) | Number of Observations | Number of Normal Points | IRV Fit RMS (meters) | High Order Polynomial RMS (meters) |
| --- | --- | --- | --- | --- |
| Ajisai (1500) |  |  |  |  |
| 2003-12-03 00:50:12 | 5169 | 23 | 0.011292 | 0.011339 |
| 2003-12-08 00:23:00 | 6643 | 27 | 0.01119 | 0.020668 |
| 2003-12-11 22:49:02 | 5916 | 24 | 0.011018 | 0.011731 |
| 2003-12-16 01:20:03 | 975 | 13 | 0.013975 | 0.013795 |
| BEC (317) |  |  |  |  |
| 2003-12-03 01:01:50 | 3275 | 35 | 0.006432 | 0.014763 |
| 2003-12-03 02:54:38 | 3482 | 36 | 0.006734 | 0.04241 |
| 2003-12-07 23:15:54 | 3388 | 28 | 0.007627 | 0.008133 |
| 2003-12-08 01:09:22 | 4527 | 41 | 0.006792 | 0.182631 |
| 2003-12-12 02:06:26 | 2595 | 20 | 0.005145 | 0.005132 |
| 2003-12-16 01:01:04 | 2531 | 27 | 0.009176 | 0.146011 |
| Champ (8002) |  |  |  |  |
| 2003-12-03 02:42:30 | 104 | 0 | 0.002192 | 0.002306 |
| 2004-01-08 00:20:02 | 627 | 0 | 0.002367 | 0.002423 |
| Envisat-1 (6179) |  |  |  |  |
| 2003-12-03 02:18:00 | 261 | 6 | 0.00336 | 0.003389 |
| 2003-12-12 02:35:58 | 2062 | 21 | 0.002911 | 0.003553 |
| 2004-01-08 01:47:28 | 2009 | 22 | 0.002929 | 0.002942 |
| 2004-01-11 01:52:58 | 2737 | 23 | 0.002709 | 0.002718 |
| ERS-2 (6178) |  |  |  |  |
| 2003-12-03 02:45:48 | 1620 | 20 | 0.003803 | 0.020042 |
| 2003-12-12 03:03:06 | 2918 | 21 | 0.003316 | 0.042852 |
| 2004-01-08 02:16:12 | 1614 | 19 | 0.003158 | 0.003187 |
| Etalon-1 (525) |  |  |  |  |
| 2003-12-07 22:45:47 | 115 | 2 | 0.01272 | 0.012816 |
| 2003-12-07 23:11:17 | 67 | 1 | 0.013535 | 0.013535 |
| 2004-01-07 23:03:53 | 212 | 5 | 0.014695 | 0.013891 |
| GFO-1 (8501) |  |  |  |  |
| 2003-12-12 03:58:48 | 3007 | 23 | 0.003016 | 0.06234 |
| Glonass-89 (9089) |  |  |  |  |
| 2003-12-02 23:53:19 | 112 | 2 | 0.010911 | 0.010822 |
| 2003-12-03 01:29:58 | 290 | 2 | 0.006274 | 0.005868 |
| 2003-12-03 01:37:42 | 52 | 2 | 0.017935 | 0.02041 |
| 2004-01-11 00:43:11 | 1215 | 7 | 0.009593 | 0.009476 |
| GPS-35 (3535) |  |  |  |  |
| 2003-12-03 01:48:58 G | 903 | 3 | 0.017576 | 0.017715 |
| 2004-01-09 23:37:25 | 2846 | 3 | 0.016782 | 0.01677 |
| Jason (4378) |  |  |  |  |
| 2003-12-02 22:03:42 | 835 | 13 | 0.003339 | 0.003329 |
| 2003-12-03 00:00:21 | 1310 | 21 | 0.003026 | 0.003007 |
| 2003-12-31 01:05:43 | 2015 | 25 | 0.00409 | 0.004088 |
| 2004-01-08 00:13:51 | 1788 | 21 | 0.003063 | 0.003047 |
| 2004-01-10 01:00:38 | 2231 | 36 | 0.003321 | 0.003309 |
| 2004-01-11 01:24:16 | 468 | 20 | 0.003402 | 0.003409 |
| Lageos-1 (1155) |  |  |  |  |
| 2003-12-03 03:07:11 | 1404 | 3 | 0.005676 | 0.005606 |
| 2003-12-03 03:11:47 | 7541 | 13 | 0.005621 | 0.005599 |
| 2003-12-12 04:49:42 | 13133 | 24 | 0.005522 | 0.005724 |
| 2003-12-16 02:54:23 | 3604 | 22 | 0.007149 | 0.006989 |
| Lageos-2 (5986) |  |  |  |  |
| 2003-12-03 04:26:23 | 10584 | 23 | 0.005996 | 0.006082 |
| 2003-12-12 03:28:31 | 5216 | 14 | 0.006204 | 0.006123 |
| 2003-12-12 04:08:13 | 207 | 2 | 0.005459 | 0.005335 |
| 2004-01-08 00:35:36 | 7150 | 19 | 0.005979 | 0.005977 |
| 2004-01-08 13:00:49 | 667 | 6 | 0.006899 | 0.006836 |
| 2004-01-10 01:19:13 | 1528 | 5 | 0.006268 | 0.006179 |
| Larets (5557) |  |  |  |  |
| 2003-12-03 04:19:38 | 264 | 5 | 0.003157 | 0.003169 |
| 2003-12-12 03:20:19 | 2698 | 25 | 0.003251 | 0.038523 |
| Stella (643) |  |  |  |  |
| 2003-12-02 21:52:30 | 747 | 8 | 0.003362 | 0.003286 |
| 2003-12-07 22:58:06 | 1862 | 20 | 0.004412 | 0.006042 |
| 2003-12-31 22:24:30 | 2170 | 22 | 0.00396 | 0.003966 |
| 2004-01-07 22:38:21 | 2504 | 25 | 0.013507 | 0.056648 |
| Topex (4377) |  |  |  |  |
| 2003-12-02 22:09:10 | 3941 | 38 | 0.01303 | 0.019016 |
| 2003-12-03 00:07:20 | 3194 | 28 | 0.010316 | 0.01027 |
| 2003-12-31 01:13:08 | 4848 | 40 | 0.013549 | 0.013631 |
| 2004-01-08 00:22:37 | 4974 | 42 | 0.012703 | 0.016186 |
| 2004-01-10 01:10:09 | 2786 | 26 | 0.008635 | 0.010074 |
| 2004-01-11 01:31:47 | 1768 | 16 | 0.007269 | 0.007184 |

The GUTS system generated 131 normal points on Lageos 1 and Lageos 2 in 10 passes tracked during the period following December 2, 2003.  The mean normal point rms across all 131 normal points (with 1 edited) was 0.51mm.  The following table provides the mean normal point rms on a pass by pass basis.

Lageos Pass Normal Point RMS

| Date of Data (GMT) | Number of Normal Points | Mean Normal Point RMS (mm) |
| --- | --- | --- |
| Lageos–1 (1155) |  |  |
| 2003-12-03 03:07:11 | 3 | 0.48 |
| 2003-12-03 03:11:47 | 13 | 0.28 |
| 2003-12-12 04:49:42 | 24 | 0.30 |
| 2003-12-16 02:54:23 | 22 | 1.10 |
| Lageos-2 (5986) |  |  |
| 2003-12-03 04:26:23 | 23 | 0.31 |
| 2003-12-12 03:28:31 | 14 | 0.50 |
| 2003-12-12 04:08:13 | 2 | 0.88 |
| 2004-01-08 00:35:36 | 19 | 0.37 |
| 2004-01-08 13:00:49 | 6 | 0.98 |
| 2004-01-10 01:19:13 | 5 | 0.40 |

The following few pages display screen shots of a selection of SPAS plots for Ajisai, Lageos 1, Lageos 2, and Topex passes.  The Topex pass is included mainly to show the receive energy dependency curve across the full range of receive energies.  Also included is a return rate plot for a GPS-35 pass acquired in geosynchronous mode.

# Collocated Data

While at GGAO, the GUTS system collocated 25 passes with MOBLAS 7 on 12 different satellites.  The following plots were generated by Polyquick and compare GUTS (Test) data with data from the reference (Base) station, MOBLAS 7.

Stella 2004-01-07 22:38:21

Champ 2004-01-08 00:20:02

Topex 2004-01-08 00:22:37

Envisat-1 2004-01-08 01:47:28

Jason 2004-01-10 01:00:38

Topex 2004-01-10 01:10:09

Lageos 2 2004-01-10 01:19:13

Glonass 89 2004-01-11 00:43:11

Jason 2004-01-10 01:00:38

Topex 2004-01-11 01:31:47

Envisat-1 2004-01-11 01:52:58

The following table lists the Polyquick results for the collocated passes.  In the table, the WTD Mean column gives the difference between the systems, GUTS minus MOBLAS 7.  MOBLAS 7 suffered a hardware failure during the last half of December.

Polyquick Results for Collocated Passes

| COSPAR | Date | Time (GMT) | Bins | GUTS Dly | M7 Dly | M7 Obs | GUTS Obs | GUTS RMS | M7 RMS | WTD Mean | Peak Peak | Diff. Temp | Dif H | WTD Press | GTS SFT | M7 SFT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | hhmm |  |  |  |  |  | mm | mm | mm | mm | C | % | mlb | mm | mm |
| 9800701 | 11/18/03 | 124 | 11 | 33304.84 | 14597.49 | 773 | 866 | 6.2 | 7.4 | 3.4 | 10.7 | 1 | 1 | 0.2 | 0 | 0.9 |
| 7501001 | 11/18/03 | 136 | 11 | 33304.84 | 14597.49 | 966 | 1705 | 11.9 | 11.1 | 5.2 | 32.8 | 1.6 | 1 | 0.2 | 0 | 0.9 |
| 105501 | 11/18/03 | 202 | 6 | 33304.84 | 14597.49 | 1557 | 413 | 346.7 | 71.1 | -0.9 | 4.7 | 0.7 | 0 | 0.1 | 0 | 0.9 |
| 9306102 | 12/2/03 | 2148 | 3 | 28214.52 | 14597.34 | 806 | 732 | 3.5 | 7.3 | -4 | 2.1 | 0.2 | 7 | 0.1 | 0 | 3.3 |
| 105501 | 12/2/03 | 2356 | 15 | 28206.12 | 14597.34 | 1322 | 1042 | 3.5 | 6.9 | -3.7 | 7 | 0.2 | 7 | 0.3 | 0 | 1.95 |
| 6503201 | 12/3/03 | 58 | 13 | 28207.62 | 14597.34 | 1348 | 3156 | 1077.8 | 666.8 | -29.3 | 518.5 | 0.1 | 6 | 0.1 | 0 | 1.95 |
| 9205201 | 12/3/03 | 6 | 14 | 28206.12 | 14597.34 | 1055 | 2555 | 33.9 | 27.7 | -14 | 14.3 | 0.1 | 7 | 0.2 | 0 | 1.95 |
| 8606101 | 12/3/03 | 48 | 17 | 28207.92 | 14597.34 | 1631 | 3595 | 14.4 | 15.3 | -13.4 | 18.6 | 0.1 | 7 | 0.2 | 0 | 1.95 |
| 9207002 | 12/3/03 | 426 | 3 | 28207.92 | 14601.54 | 84 | 625 | 6.8 | 8.7 | -2.2 | 2.5 | 1.6 | 9 | 0.2 | 0 | 0 |
| 9306102 | 12/7/03 | 2256 | 8 | 28207.77 | 14600.19 | 1170 | 1260 | 631.1 | 608.9 | -12 | 607.7 | 0.5 | 11 | 0.2 | 0.75 | -1.35 |
| 8606101 | 12/8/03 | 20 | 2 | 28208.52 | 14599.74 | 337 | 291 | 10.9 | 14.1 | -3.2 | 0.3 | 1.3 | 12 | 0.2 | 1.35 | 2.25 |
| 6503201 | 12/8/03 | 108 | 34 | 28208.67 | 14600.49 | 1993 | 4471 | 614 | 575.3 | -12.3 | 397.5 | 2.9 | 7 | 0.2 | 0 | -0.45 |
| 7603901 | 12/16/03 | 254 | 1 | 28208.52 | 14592.85 | 60 | 15 | 5.9 | 13.8 | 2.2 | 0 | 2.6 | 3 | 0.1 | 5.25 | 4.95 |
| 8606101 | 12/16/03 | 118 | 12 | 28204.47 | 14589.25 | 1195 | 964 | 14.4 | 17.5 | -8 | 11.7 | 0.1 | 5 | 0.2 | 0.45 | 4.8 |
| 9306102 | 1/7/04 | 2239 | 11 | 28193.7 | 14590 | 381 | 780 | 6.4 | 12.8 | -5.5 | 15.7 | 7 | 9 | 0.25 | 0 | 1.95 |
| 3902 | 1/8/04 | 20 | 7 | 28194.3 | 14593.3 | 123 | 288 | 4.8 | 15.3 | -20.9 | 14.6 | 7.8 | 9 | 0.2 | 0.3 | 1.05 |
| 9205201 | 1/8/04 | 23 | 15 | 28194.3 | 14593.3 | 783 | 1426 | 13.2 | 19.2 | -6.9 | 20.7 | 7.7 | 9 | 0.23 | 0.3 | 1.5 |
| 200901 | 1/8/04 | 147 | 4 | 28195.5 | 14594.8 | 139 | 166 | 6.6 | 18 | 1.7 | 10.5 | 8.6 | 9 | 0.2 | 0.6 | 4.8 |
| 105501 | 1/10/04 | 101 | 23 | 28198.8 | 14579.5 | 699 | 1719 | 4.2 | 11.9 | -21.8 | 11.2 | 11.8 | 7 | 0.14 | 1.65 | 8.5 |
| 9205201 | 1/10/04 | 110 | 21 | 28198.3 | 14579.5 | 828 | 2190 | 9 | 15.9 | -31.7 | 13.5 | 12 | 7 | 0.1 | 1.65 | 8.5 |
| 9207002 | 1/10/04 | 119 | 4 | 28197.9 | 14579.5 | 194 | 1218 | 7.8 | 12.3 | 4.5 | 9.6 | 12.5 | 8 | 0.17 | 1.65 | 8.5 |
| 206001 | 1/11/04 | 43 | 2 | 28193.7 | 23798.3 | 132 | 298 | 12.3 | 15.3 | 5.8 | 2.4 | 20 | 6 | 0.1 | 0 | -4.05 |
| 105501 | 1/11/04 | 124 | 6 | 28193.7 | 14577.3 | 82 | 227 | 4.1 | 9.5 | -20 | 4.6 | 24 | 8 | 0.1 | 0 | 1.05 |
| 9205201 | 1/11/04 | 132 | 5 | 28193.7 | 14577.3 | 621 | 1637 | 8 | 13.2 | -23.6 | 16.7 | 23.9 | 14 | 0.1 | 0 | 1.05 |
| 200901 | 1/11/04 | 153 | 12 | 28193.7 | 14577.3 | 499 | 1528 | 3.4 | 10.3 | 6.3 | 5.1 | 23.5 | 17 | 0.2 | 0 | 1.05 |